MESA Renal Ancillary Study CT Data

MESA Renal Methodology

MESA Renal Volumes and HU measurements: The measurements were performed by a CT reader, blinded to the patient characteristics, using abdominal CT images. All renal volume measurements were performed on the axial slices. Each kidney volume was measured as a sum of three separate areas: superior, mid and inferior segments. The superior and inferior segments were differentiated from mid segment by the absence of renal pelvis on the images. We manually traced the renal borders and excluded renal pelvis from the mid segment measurements. For each segment, we measured volume as well as average HU attenuation. The average HU of each renal segment was calculated from the HU values of the individual slices of each kidney segment. The total kidney volume will be the sum of each of these superior, mid and inferior segments.

Renal Artery Calcification: Renal artery calcification (RAC) was measured using previously published Agatston method. RAC was defined as a plaque of at least three contiguous pixels (area ≥1.02 mm2) with a density of >130 Hounsfield units. The lesion score was calculated by multiplying the lesion area by a density factor derived from the maximal Hounsfield unit within this area, as described by Agatston et al. The density factor were derived in the following manner: 1 for lesions with peak attenuation of 130–199, 2 for lesions with peak attenuation of 200–299, 3 for lesions with peak attenuation of 300–399, and 4 for lesions with peak attenuation of >400. We measured renal artery ostial calcification and calcification present in the arterial segment proper. Calcium in the wall of the abdominal aorta was not included in the assessment for RAC by excluding any visible calcium in the extrapolated plane of the aorta at the renal artery origin. For each individual artery, the RAC score will be sum of both ostial and arterial segment proper calcification.

Renal Artery Diameter: Renal artery diameters were measured for each of the renal artery on axial planes at two levels: 1-2mm from the ostium and 1-2mm prior to entrance into the renal pelvis.

Data: A blank cell represents missing data. A zero (0) represents a measured value of 0.

MESA Ancillary Study 86 Data Set Variable Guide

Exam 2 and 3 Renal CT Dataset

| Data Set name : | MESAe23a84_RenalCT_20131017 |
| --- | --- |
| as86 Principal Investigator : | Matthew Allison |
| as86 Contact Information : | mallison@ucsd.edu |

| Order | Variable | Variable Description | Value Labels |
| --- | --- | --- | --- |
| 1 | idno | MESA Participant ID |  |
| 2 | ctexam | Exam Corresponding to CT Scan | 2: Exam 2 3: Exam 3 |
| 3 | ctrscntt | Time from Baseline Exam to CT Scan |  |
| 4 | compkidneyspresent3 | Indicates whether both Kidneys are completely imaged | 0: No 1: Yes |
| 5 | comprapresent3 | Indicates whether both Renal Arteries are completely images | 0: No 1: Yes |
| 6 | compleftpelvis3 | Left Pelvis Completed | 0: No 1: Yes |
| 7 | comprightpelvis3 | Right Pelvis Completed | 0: No 1: Yes |
| 8 | lvolumesuperior3 | Left Kidney Volume (Superior Segment) - mm3 |  |
| 9 | lvolumemid3 | Left Kidney Volume (Mid Segment) - mm3 |  |
| 10 | lvolumeinferior3 | Left Kidney Volume (Inferior Segment) - mm3 |  |
| 11 | rvolumesuperior3 | Right Kidney Volume (Superior Segment) - mm3 |  |
| 12 | rvolumemid3 | Right Kidney Volume (Mid Segment) - mm3 |  |
| 13 | rvolumeinferior3 | Right Kidney Volume (Inferior Segment) - mm3 |  |
| 14 | ldensitysuperiorhu3 | Left Kidney Mean HU (Superior Segment) |  |
| 15 | ldensitysuperiorsd3 | Left Kidney Standard Deviation (Superior Segment) |  |
| 16 | ldensitymidhu3 | Left Kidney Mean HU (Mid Segment) |  |
| 17 | ldensitymidsd3 | Left Kidney Standard Deviation (Mid Segment) |  |
| 18 | ldensityinferiorhu3 | Left Kidney Mean HU (Inferior Segment) |  |
| 19 | ldensityinferiorsd3 | Left Kidney Standard Deviation (Inferior Segment) |  |
| 20 | rdensitysuperiorhu3 | Right Kidney Mean HU (Superior Segment) |  |
| 21 | rdensitysuperiorsd3 | Right Kidney Standard Deviation (Superior Segment) |  |
| 22 | rdensitymidhu3 | Right Kidney Mean HU (Mid Segment) |  |
| 23 | rdensitymidsd3 | Right Kidney Standard Deviation (Mid Segment) |  |
| 24 | rdensityinferiorhu3 | Right Kidney Mean HU (Inferior Segment) |  |
| 25 | rdensityinferiorsd3 | Right Kidney Standard Deviation (Inferior Segment) |  |
| 26 | lradiameter3 | Left Renal Artery Proximal Diameter |  |
| 27 | lradistdiameter3 | Left Renal Artery Distal Diameter |  |
| 28 | rradiameter3 | Right Renal Artery Proximal Diameter |  |
| 29 | rradistdiameter3 | Right Renal Artery Distal Diameter |  |
| 30 | lrocagatstonsc3 | Left Kidney Renal Ostia Agatston Score |  |
| 31 | lrocvolumesc3 | Left Kidney Renal Ostia Volume Score |  |
| 32 | lracagatstonsc3 | Left Kidney Renal Artery Agatston Score |  |
| 33 | lracvolumesc3 | Left Kidney Renal Artery Volume Score |  |
| 34 | rrocagatstonsc3 | Right Kidney Renal Ostia Agatston Score |  |
| 35 | rrocvolumesc3 | Right Kidney Renal Ostia Volume Score |  |
| 36 | rracagatstonsc3 | Right Kidney Renal Artery Score |  |
| 37 | rracvolumesc3 | Right Kidney Renal Volume Score |  |
| 38 | volumereader3 | Volume Reader |  |
| 39 | canotes3 | Notes |  |