MESA Sleep Heart Rate Variability Analysis
December 15, 2017

Pre-processing and related considerations

- Individuals with less than 1,000 normal-to-normal (NN) intervals and/or less than two hours of combined sleep stages REM, N1, N2, N3 and N4 were excluded.

- Sleep onset was defined as the start time of the first non-awake period longer than 90-seconds. Similarly, sleep termination was defined as the end time of the last non-awake period longer than 90-second.

- NN intervals < 0.35 sec or > 2.5 were excluded.

- 5-min windows with less than 180 NN intervals were excluded.

- Two sets of HRV analyses were performed: 1) the full night was analyzed as a single period; 2) the analysis was performed in non-overlapping 5-minute windows and the resulting values were averaged.

Time Domain Measures

- NN_RR - Ratio of consecutive normal sinus beats (NN) over all cardiac inter-beat (RR) intervals

- AVNN  – Average of all NN intervals. Units: millisecond (ms).

- IHR – Average instantaneous heart rate. For a given NN interval, IHR=60/NN. Units: bpm (beat per minute)

- SDNN – Standard deviation of all NN intervals. Units: ms.

- SDANN – Standard deviation of the averages of NN intervals in all 5-minute segments. Units: ms.

- SDNNIDX – Mean of the standard deviations of NN intervals in all 5-minute segments.Units: ms.

- rMSSD – Square root of the mean of the squares of differences between adjacent NN intervals. Units: ms.

- pNN10 – Percentage of differences between adjacent NN intervals that are greater than 10 ms; a member of the larger pNNx family. Unitless.

- pNN20 – Percentage of differences between adjacent NN intervals that are greater than 20 ms; a member of the larger pNNx family. Unitless.

- pNN30 – Percentage of differences between adjacent NN intervals that are greater than 30 ms; a member of the larger pNNx family. Unitless.

- pNN40 – Percentage of differences between adjacent NN intervals that are greater than 40 ms; a member of the larger pNNx family. Unitless.

- pNN50 – Percentage of differences between adjacent NN intervals that are greater than 50 ms; a member of the larger pNNx family. Unitless.

Frequency Domain Measures

- TOTPWR  –Total spectral power of all NN intervals up to 0.04 Hz. Units: milliseconds square (ms2).

- ULF  – Total spectral power of all NN intervals up to 0.003 Hz (not available for analysis using 5-minute windows). Units: ms2.

- VLF  – Total spectral power of all NN intervals between 0.003 and 0.04 Hz, and between 0 and 0.04 Hz in analyses of the entire night period and 5-minute windows, respectively. Units: ms2.

- LF  – Total spectral power of all NN intervals between 0.04 and 0.15 Hz. Units: ms2.

- HF  – Total spectral power of all NN intervals between 0.15 and 0.4 Hz Units: ms2.

- LF/HF  – Ratio of low to high frequency power. Unitless.

- HFn – Normalized HF, ratio HF/(HF+LF). Unitless.

- LFn – Normalized LF, ratio LF/(HF+LF). Unitless.

Reference: Software and tutorial available at https://www.physionet.org/tutorials/hrv-toolkit/